DISTRIBUTED DEEP LEARNING

PLAYBOOK

From Single GPU to Multi-Node Training

A Hands-On Guide with PyTorch DDP, NCCL, NVLink & Profiling

Rustem Kamun

March 2026

Tested on: 2x RTX 3090 (NVLink) | Vast.ai

# Table of Contents

1. GPU Fundamentals — SMs, CUDA cores, key metrics, essential commands

2. Communication Backends — NCCL vs Gloo vs MPI, measured AllReduce performance

3. GPU Interconnects — NVLink, NVSwitch, PCIe hierarchy, inference throughput impact

4. Multi-Node Networking — RDMA, InfiniBand, RoCE, TCP tuning, technology hierarchy

5. Distributed Data Parallel (DDP) — Overlap mechanics, measured results, pitfalls

6. The Training Loop — Forward/backward/optimizer anatomy, zero_grad, gradient accumulation

7. Launching Distributed Training — torchrun vs mp.spawn, multi-node commands

8. Profiling — nsys, PyTorch profiler, trace flags, report analysis, SQLite queries

9. Cloud GPU Setup (Vast.ai) — Instance selection, verification, cost comparison

10. LLM Inference with vLLM — Tensor parallelism, serving setup, testing

11. Attention, KV-Cache & Inference Communication — QKV mechanics, cache sizing, GQA, disaggregated serving

12. GPU Monitoring — nvidia-smi dmon, reading utilization patterns, what to watch for

13. vLLM Inference Under Load — Load testing, measured patterns, capacity planning

Appendix — Quick reference card, hardware specifications

# 1. GPU Fundamentals

Understanding GPU architecture is the foundation for optimizing distributed training. Your RTX 3090 has 82 Streaming Multiprocessors (SMs), each containing 128 CUDA cores, totaling 10,496 CUDA cores with 24GB GDDR6X VRAM.

## 1.1 Key GPU Metrics

| Metric | What It Measures | How to Check |
| --- | --- | --- |
| SM Utilization | % of streaming multiprocessors actively computing | nvidia-smi, nsys profile |
| GPU-Util | Time-averaged SM utilization (sampled) | nvidia-smi (GPU-Util column) |
| Memory Usage | VRAM allocated vs total | nvidia-smi (Memory-Usage) |
| Power Draw | Current vs max TDP | nvidia-smi (Pwr:Usage/Cap) |
| Perf State | P0 = full performance, P8 = idle | nvidia-smi (Perf column) |

## 1.2 Essential Commands

nvidia-smi                              # Snapshot of GPU state

watch -n 1 nvidia-smi                   # Live monitoring (1s refresh)

nvidia-smi topo -m                      # GPU interconnect topology

nvidia-smi nvlink -s                    # NVLink status and link speeds

nvtop                                   # Interactive GPU monitor (apt install nvtop)

# 2. Communication Backends

PyTorch supports three backends for distributed communication. The choice depends on your hardware.

| Backend | Transport | Best For | Speed |
| --- | --- | --- | --- |
| NCCL | GPU-direct (NVLink, PCIe, RDMA) | Multi-GPU training (production) | Fastest for GPUs |
| Gloo | CPU memory + TCP/shared memory | CPU training, single-GPU multi-rank simulation | Slower (CPU round-trip) |
| MPI | Network (TCP, InfiniBand) | Legacy HPC clusters | Varies |

## 2.1 Backend Selection Logic

if not torch.cuda.is_available() or torch.cuda.device_count() < world_size:

backend = 'gloo'    # Fallback: single GPU or CPU-only

else:

backend = 'nccl'    # Production: one GPU per rank

## 2.2 NCCL Bootstrap Process

When init_process_group() is called with NCCL, two phases occur:

- Bootstrap (TCP): All ranks exchange metadata (GPU IDs, topology, memory handles) over regular TCP using MASTER_ADDR:MASTER_PORT as the rendezvous point. Each rank also opens ephemeral TCP ports for peer connections.

- Data plane (GPU-direct): After bootstrap, NCCL sets up fast paths over NVLink, PCIe, or RDMA. All actual gradient AllReduce uses these channels. TCP is no longer involved.

On large clusters (100+ GPUs), the bootstrap phase can exhaust ephemeral TCP ports. Widen the range with: echo '20000 65535' > /proc/sys/net/ipv4/ip_local_port_range

## 2.3 Measured AllReduce Performance

AllReduce of model gradients (~2M parameters) across 2 ranks:

| Setup | AllReduce Time | Speedup |
| --- | --- | --- |
| Single GPU, Gloo (CPU round-trip) | 13,166 µs (13.2 ms) | 1x (baseline) |
| 2x RTX 3090, NCCL + NVLink | 296 µs (0.3 ms) | ~28x faster |
| 2x RTX 3090, NCCL + NVLink (DDP) | 460 µs (0.46 ms) | ~28x faster |

# 3. GPU Interconnects

## 3.1 Interconnect Hierarchy

| Interconnect | Bandwidth | Latency | Description |
| --- | --- | --- | --- |
| NVLink (RTX 3090) | 56 GB/s per direction | Low | Direct GPU-to-GPU physical bridge, bypasses CPU |
| PCIe 4.0 x16 | 32 GB/s | Medium | Standard bus, data goes through CPU chipset |
| PCIe 3.0 x16 | 16 GB/s | Medium | Older standard, half PCIe 4.0 bandwidth |
| Gloo (CPU) | ~10-15 GB/s | High | GPU → CPU RAM → reduce → CPU RAM → GPU |

## 3.2 NVLink Verification

# Check topology (NV# = NVLink, PIX/PHB = PCIe)

nvidia-smi topo -m

# Check link speed (RTX 3090: 4 links x 14 GB/s = 56 GB/s)

nvidia-smi nvlink -s

Data path comparison:

- Gloo: GPU → PCIe → CPU memory → CPU reduce → CPU memory → PCIe → GPU

- NCCL + NVLink: GPU → NVLink bridge → GPU (direct, no CPU involvement)

## 3.3 NVLink vs NVSwitch

NVLink connects GPUs in pairs via a physical bridge (what your 2x 3090 setup uses). NVSwitch is a separate chip that acts as a central fabric connecting ALL GPUs in a server simultaneously:

| Feature | NVLink (Point-to-Point) | NVSwitch |
| --- | --- | --- |
| Topology | Direct bridge between 2 GPUs | Central switch connecting all GPUs |
| Bandwidth | 128 GB/s (H100 pair) | 900 GB/s (H100 NVSwitch) |
| Scaling | Each GPU needs a separate link per peer | Any GPU talks to any GPU at full speed |
| AllReduce | Ring pattern through links | All GPUs reduce simultaneously via NVLS |
| Cost | Included on GPU card | Separate chip, adds system cost |

The impact on total processing time: with point-to-point NVLink, there is a visible 'Comms' block after compute in each training/inference step. With NVSwitch's 7x higher bandwidth, that communication block shrinks dramatically — making the total processing time shorter.

## 3.4 NVSwitch Impact on Inference Throughput

Measured Llama 3.1 70B inference throughput at various real-time response targets (tokens/second per user). The numbers in parentheses are batch sizes — how many concurrent users can be served while meeting the latency target:

| Response Budget | Single GPU (TP=1) | Point-to-Point (TP=2) | NVSwitch (TP=2) | NVSwitch Benefit |
| --- | --- | --- | --- | --- |
| 30 tok/s/user | 67 tok/s (batch 2) | 80 tok/s (batch 6) | 115 tok/s (batch 9) | 1.4x |
| 35 tok/s/user | Does Not Meet | 74 tok/s (batch 5) | 104 tok/s (batch 7) | 1.4x |
| 40 tok/s/user | Does Not Meet | 67 tok/s (batch 4) | 87 tok/s (batch 5) | 1.3x |
| 45 tok/s/user | Does Not Meet | 56 tok/s (batch 3) | 76 tok/s (batch 4) | 1.4x |
| 50 tok/s/user | Does Not Meet | 43 tok/s (batch 2) | 63 tok/s (batch 3) | 1.5x |

## 3.5 Reading the Throughput Table

The response budget is the minimum tokens per second each user must receive — how fast text appears on screen. The GPU must divide its total throughput across all concurrent users:

Example: Single GPU, 30 tok/s budget, batch size 2

GPU total output:  67 tok/s

Serving 2 users:   67 / 2 ≈ 33 tok/s per user

Budget: 30 tok/s   → 33 > 30 ✓ Meets requirement

Why not batch size 3?

67 / 3 ≈ 22 tok/s per user

Budget: 30 tok/s   → 22 < 30 ✗ Too slow for users

NVSwitch enables higher batch sizes at the same latency target because faster interconnect means less time spent on AllReduce between layers. At 30 tok/s budget: point-to-point serves 6 users, NVSwitch serves 9 users — that is 1.5x more users from the same hardware, which directly translates to 1.5x more revenue for inference providers.

At stricter response targets (50 tok/s), the NVSwitch benefit increases to 1.5x because there is less compute time available to hide the communication latency — the faster interconnect becomes more critical.

# 4. Multi-Node Networking

When training spans multiple machines, the network becomes the bottleneck. Understanding these technologies is critical for scaling.

## 4.1 Network Technologies

| Technology | What It Does | Bandwidth |
| --- | --- | --- |
| Ethernet (standard) | TCP/IP networking, CPU manages packets | 10-100 Gbps |
| RoCE (RDMA over Converged Ethernet) | RDMA over Ethernet hardware, needs compatible NICs | 100-400 Gbps |
| InfiniBand | Purpose-built RDMA fabric, lowest latency | 200-400+ Gbps |
| GPUDirect RDMA | GPU memory → NIC → wire → NIC → GPU memory | Line rate |

## 4.2 Key Concepts

DMA (Direct Memory Access): Hardware engine on a device that handles data transfer independently. The CPU just sets up source, destination, and size, then the DMA engine completes the transfer without further CPU involvement.

RDMA (Remote DMA): DMA across the network between two machines. One machine's NIC reads directly from memory and writes into the other machine's memory. No CPU involvement on either side.

NVMe (Non-Volatile Memory Express): Fast storage protocol connecting SSDs directly via PCIe bus (3-7 GB/s vs SATA's 550 MB/s).

GPUDirect Storage: GPU's DMA engine pulls data directly from NVMe SSD into GPU memory, skipping CPU memory entirely.

## 4.3 TCP Tuning for Training

- Jumbo frames (MTU 9000): 6x fewer packets, 6x less CPU interrupt overhead

- Buffer sizes: Set rmem_max/wmem_max large enough to fill the bandwidth-delay product

- Congestion control: CUBIC (default) works on dedicated clusters; BBR better for shared/high-latency links

# Check/set MTU

ip link show eth0

# Check TCP buffers

sysctl net.core.rmem_max

# Check congestion algorithm

sysctl net.ipv4.tcp_congestion_control

## 4.4 Check Your NICs

ip link show              # List interfaces

lspci | grep -i net       # NIC hardware

rdma link show            # RDMA capability

ethtool eth0 | grep Speed # Link speed

## 4.5 Complete Technology Hierarchy

All interconnect and data movement technologies organized by scope:

| Scope | Technology | Data Path | Use Case |
| --- | --- | --- | --- |
| Within a machine | NVLink | GPU → GPU directly | Multi-GPU training/inference (your 2x 3090) |
| Within a machine | GPUDirect Storage | Local NVMe SSD → GPU | Fast dataset loading, skips CPU |
| Across machines | InfiniBand | GPU → NIC → NIC → GPU (RDMA) | Production clusters, lowest latency |
| Across machines | RoCE | GPU → NIC → NIC → GPU (RDMA over Ethernet) | Clusters with existing Ethernet infra |
| Across machines | TCP (fallback) | GPU → CPU → NIC → NIC → CPU → GPU | Basic networking, highest latency |
| Remote storage | NVMe-oF (NVMe over Fabrics) | Remote SSD → NIC → GPU | Access remote SSDs as if local |

## 4.6 Optimization Layers

GPUDirect RDMA and GPUDirect Storage are optimizations that layer on top of the network/storage technologies to eliminate CPU involvement:

| Without Optimization | With Optimization | What It Eliminates |
| --- | --- | --- |
| GPU → CPU → NIC → wire | GPU → NIC → wire (GPUDirect RDMA) | CPU copy on send side |
| wire → NIC → CPU → GPU | wire → NIC → GPU (GPUDirect RDMA) | CPU copy on receive side |
| SSD → CPU memory → GPU | SSD → GPU (GPUDirect Storage) | CPU memory round-trip |

## 4.7 Communication Libraries

Different libraries are optimized for different workload patterns:

| Library | Designed For | Key Strength |
| --- | --- | --- |
| NCCL | Training (collectives: AllReduce, Broadcast) | Bulk gradient sync across all GPUs simultaneously |
| UCX | General-purpose RDMA | Thin layer, low overhead for point-to-point transfers |
| NIXL | Inference (point-to-point data movement) | KV-cache transfer, disaggregated serving, dynamic scaling |
| Gloo | CPU training, prototyping | Works everywhere, no special hardware needed |

# 5. Distributed Data Parallel (DDP)

## 5.1 How DDP Works

DDP wraps a model and hooks into backward() to overlap gradient AllReduce with gradient computation. This is the key optimization over manual AllReduce.

- Broadcast: At DDP init, rank 0's weights are broadcast to all ranks (ensures identical starting point)

- AllGather: DDP collects metadata from all ranks to agree on gradient bucket partitioning

- AllReduce: During backward(), as each gradient bucket fills, DDP fires AllReduce immediately (overlaps with ongoing gradient computation)

- Optimizer: After backward completes, all ranks have identical averaged gradients and take the same optimizer step

## 5.2 No-Overlap vs DDP Overlap (Measured)

| Phase | No-Overlap (Sequential) | DDP Overlapped | Notes |
| --- | --- | --- | --- |
| Forward | 123 ms | 124 ms | Same (no communication) |
| Backward | 71 ms | ─┐ |  |
| AllReduce | 10.9 ms (296µs CUDA) | ─┘ 80.6 ms | AllReduce hidden inside backward |
| Optimizer | 17 ms | 17 ms | Same (no communication) |
| TOTAL | 222 ms | 222 ms | Tiny model = minimal overlap benefit |

The overlap benefit is small here because the model (3 layers, ~2M params) is tiny. With a large transformer (hundreds of layers), AllReduce would take significant time that can be hidden behind backward computation.

## 5.3 Critical DDP Pitfalls

- Each rank MUST map to its own GPU: device = f'cuda:{rank}' + torch.cuda.set_device(rank)

- NCCL deadlocks if multiple ranks share cuda:0 (the bug we fixed in the PR)

- Use gradient_as_bucket_view=True to reduce memory copies

- DDP already does AllReduce; do NOT add manual all_reduce on top (double reduction)

- Always call optimizer.zero_grad() at the start of each training step

# 6. The Training Loop

## 6.1 Anatomy of a Training Step

Each training step consists of four phases that repeat thousands of times:

- Forward pass: Model makes predictions, compute loss (how wrong it was)

- Backward pass: Compute gradients (direction + magnitude to adjust each weight)

- AllReduce: Average gradients across all ranks (DDP does this during backward)

- Optimizer step: Actually update weights using gradients (e.g., SGD: weight -= lr * grad)

## 6.2 Key Details

zero_grad(): PyTorch accumulates gradients by default. Each backward() adds to .grad rather than replacing it. Call optimizer.zero_grad() before each step to reset.

Gradient accumulation: The accumulation behavior is useful for simulating large batch sizes with limited GPU memory. Run multiple forward+backward passes, then one optimizer.step().

# Standard training loop

for epoch in range(num_epochs):

for data, target in dataloader:

optimizer.zero_grad()

output = model(data)

loss = loss_fn(output, target)

loss.backward()

optimizer.step()

# 7. Launching Distributed Training

## 7.1 torchrun vs mp.spawn

| Feature | torchrun | mp.spawn |
| --- | --- | --- |
| Process creation | Separate OS processes | Python multiprocessing fork/spawn |
| Multi-node | Yes (--nnodes, --node_rank) | No (single machine only) |
| Env variables | Auto-sets RANK, WORLD_SIZE, etc. | You pass rank as function arg |
| Profiling (nsys) | Works out of the box | nsys can't follow child processes easily |
| Production use | Standard | Mostly for prototyping |

## 7.2 torchrun Commands

# Single node, 2 GPUs

torchrun --nproc_per_node=2 train.py

# Multi-node (2 nodes, 2 GPUs each = 4 ranks)

# Machine 1:

torchrun --nproc_per_node=2 --nnodes=2 --node_rank=0 \

--master_addr=192.168.1.100 --master_port=29500 train.py

# Machine 2:

torchrun --nproc_per_node=2 --nnodes=2 --node_rank=1 \

--master_addr=192.168.1.100 --master_port=29500 train.py

# 8. Profiling

## 8.1 Nsight Systems (nsys)

NVIDIA's system-wide profiler. Best for seeing the full timeline of CPU + GPU activity, kernel launches, memory transfers, and NCCL operations.

# Profile with auto PyTorch annotations (requires nsys 2024.5+)

nsys profile \

--trace=cuda,nvtx,osrt \

--pytorch=autograd-shapes-nvtx \

--force-overwrite=true \

--output=profile_output \

torchrun --nproc_per_node=2 train.py

## 8.2 nsys Trace Flags

| Flag | What It Captures |
| --- | --- |
| --trace=cuda | CUDA kernel launches, memory copies, synchronization |
| --trace=nvtx | NVTX range markers (manual or auto-annotated PyTorch ops) |
| --trace=osrt | OS runtime: file I/O, memory mapping, pthread operations |
| --pytorch=autograd-shapes-nvtx | Auto-annotates every PyTorch op with tensor shapes (no code changes needed) |

## 8.3 Analyzing Reports

# Full summary

nsys stats profile_output.nsys-rep

# GPU kernel breakdown (most important for training)

nsys stats --report cuda_gpu_kern_sum profile_output.nsys-rep

# CUDA API calls (CPU-side overhead)

nsys stats --report cuda_api_sum profile_output.nsys-rep

# Memory transfers

nsys stats --report cuda_gpu_mem_size_sum profile_output.nsys-rep

# NVTX ranges (labeled phases)

nsys stats --report nvtx_sum profile_output.nsys-rep

## 8.4 SQLite Export for Custom Analysis

nsys export --type=sqlite profile_output.nsys-rep

sqlite3 profile_output.sqlite

-- Communication vs Compute breakdown

SELECT

CASE WHEN name LIKE '%nccl%' THEN 'NCCL' ELSE 'Compute' END AS category,

SUM(end - start) / 1e6 AS total_ms,

COUNT(*) AS calls

FROM CUPTI_ACTIVITY_KIND_KERNEL GROUP BY category;

## 8.5 PyTorch Profiler (Alternative)

Works without nsys installation. Export chrome traces and view in chrome://tracing:

prof.export_chrome_trace('trace_rank0.json')

# scp to laptop, open chrome://tracing, load the file

## 8.6 What to Look For in Profiles

- cuda_gpu_kern_sum: NCCL % vs compute % tells you communication overhead

- cuda_api_sum: High cudaDeviceSynchronize means GPU idle time (CPU waiting)

- cuda_gpu_mem_size_sum: DtoH transfers indicate data going through CPU (bad for NCCL)

- First iteration is always slow (lazy loading); profile iteration 100+ for steady-state

# 9. Cloud GPU Setup (Vast.ai)

## 9.1 Choosing an Instance

- Filter by GPU count (2x, 4x, 8x) and model (RTX 3090, A100, H100)

- Check PCIe version: 4.0 is 2x bandwidth of 3.0

- DLPerf score: higher = better actual deep learning throughput

- Reliability: avoid anything below 99%

- Look for NVMe storage (4000+ MB/s) for dataset loading

## 9.2 After SSH-ing In

# Verify GPU setup

nvidia-smi

nvidia-smi topo -m           # Check NVLink (NV#) vs PCIe (PIX/PHB)

nvidia-smi nvlink -s          # NVLink bandwidth per link

# Check PyTorch CUDA

python -c "import torch; print(torch.cuda.device_count(), torch.distributed.is_nccl_available())"

## 9.3 Cost-Effective Options

| Instance | Use Case | Approx Cost |
| --- | --- | --- |
| 2x RTX 3090 | Learning DDP, NVLink experiments | $0.28-0.35/hr |
| 2x A100 40GB | Real multi-GPU training with NVLink | $1.50-2.00/hr |
| 4x A100 80GB | Large model training, NVLink + NVSwitch | $4-6/hr |

# 10. LLM Inference with vLLM

vLLM provides production-grade LLM serving with tensor parallelism across multiple GPUs.

## 10.1 Quick Start

pip install vllm

# Serve with tensor parallelism across 2 GPUs

vllm serve TinyLlama/TinyLlama-1.1B-Chat-v1.0 \

--tensor-parallel-size 2 \

--dtype float16 \

--max-model-len 2048 \

--port 8000

## 10.2 Testing

curl http://localhost:8000/v1/chat/completions \

-H 'Content-Type: application/json' \

-d '{"model": "TinyLlama/TinyLlama-1.1B-Chat-v1.0",

"messages": [{"role": "user", "content": "What is NVLink?"}],

"max_tokens": 200}'

## 10.3 What Happens Under the Hood

- Model weights split across GPUs (tensor parallelism: attention heads + MLP divided)

- Each forward pass involves NVLink AllReduce to combine partial results

- vLLM uses PagedAttention for efficient KV-cache memory management

- FlashAttention fused kernel avoids materializing full attention matrix

- NCCL handles cross-GPU communication transparently

# 11. Attention, KV-Cache & Inference Communication

Understanding the attention mechanism and KV-cache is essential for reasoning about inference performance, GPU memory, and why distributed inference requires fast data movement.

## 11.1 Multi-Head Attention

Every token entering a transformer layer gets transformed into three vectors: Query (Q), Key (K), and Value (V). The model splits these across multiple attention heads, each learning different relationships between tokens.

| Concept | What It Is | Analogy |
| --- | --- | --- |
| hidden_dim | Size of each token's vector representation (e.g., 2048) | Width of the model's thinking capacity |
| num_heads | Number of parallel attention computations (e.g., 32) | Multiple reviewers reading the same text |
| head_dim | hidden_dim / num_heads (e.g., 64) | Each reviewer's focused slice |
| Query (Q) | What am I looking for? | A question about context |
| Key (K) | What do I contain? | A label describing each token |
| Value (V) | What information do I carry? | The actual content to retrieve |

The full token vector (hidden_dim=2048) is split into 32 heads of 64 values each. Each head independently computes attention, then results are concatenated back to 2048. This split is just a reshape — the total data is the same.

## 11.2 How Attention Works (Step by Step)

To predict the next token, the model computes attention scores between the new token's Query and every previous token's Key, then takes a weighted sum of Values:

Sentence: 'The cat sat'

Step 1: Compute Q, K, V for each token using weight matrices

Q_sat = embedding_sat × W_q

K_the = embedding_the × W_k,  K_cat = embedding_cat × W_k, ...

V_the = embedding_the × W_v,  V_cat = embedding_cat × W_v, ...

Step 2: Score each previous token (dot product of Q with each K)

score('The') = Q_sat · K_the = 0.2  (low relevance)

score('cat') = Q_sat · K_cat = 0.7  (high relevance)

score('sat') = Q_sat · K_sat = 0.5  (medium relevance)

Step 3: Softmax → weights, then weighted sum of Values

output = 0.1 × V_the + 0.5 × V_cat + 0.4 × V_sat

## 11.3 Why Cache K and V?

LLM inference is autoregressive — each new token depends on all previous tokens. Without caching, you'd recompute K and V for every previous token at every step:

| Approach | Token 4 | Token 5 | Token 100 |
| --- | --- | --- | --- |
| Without cache | Compute Q,K,V for tokens 1-4 | Recompute Q,K,V for tokens 1-5 | Recompute Q,K,V for tokens 1-100 |
| With KV-cache | Compute Q4,K4,V4. Cache K4,V4 | Compute Q5,K5,V5. Append to cache | Compute Q100,K100,V100. Append |

With the cache, only the new token's Q, K, V are computed. Previous K's and V's are read from cache. Q doesn't need caching because you only ever need the current token's query.

## 11.4 KV-Cache Size Calculation

Example: TinyLlama 1.1B (the model we deployed on vLLM):

hidden_dim = 2048, num_heads = 32, head_dim = 64, num_layers = 22

Per token, per layer:

K vector = head_dim × num_heads = 64 × 32 = 2048 values

V vector = head_dim × num_heads = 64 × 32 = 2048 values

Total = (2048 + 2048) × 2 bytes (float16) = 8 KB

Per token (all layers) = 8 KB × 22 layers = 176 KB

| Context Length | KV-Cache Size (TinyLlama) | KV-Cache Size (Llama 70B) | Impact |
| --- | --- | --- | --- |
| 100 tokens | 17.6 MB | 32 MB | Fits easily in GPU memory |
| 1,000 tokens | 176 MB | 320 MB | Significant VRAM usage |
| 10,000 tokens | 1.76 GB | 3.2 GB | Large chunk of 24GB GPU |
| 100,000 tokens | 17.6 GB | 32 GB | Exceeds single GPU! |

## 11.5 GQA (Grouped Query Attention)

Newer models like Llama 70B noticed many heads learn similar K and V patterns. GQA shares K/V heads across groups of Q heads, dramatically reducing cache size:

Standard attention (TinyLlama):  32 Q heads, 32 K heads, 32 V heads

GQA (Llama 70B):                 64 Q heads, 8 K heads, 8 V heads

(8 groups of 8 Q heads share one KV)

Saves ~4x KV-cache memory!

## 11.6 Inference Communication Patterns

Different model parallelism strategies create different communication patterns during inference:

| Strategy | Communication Pattern | What Moves | When |
| --- | --- | --- | --- |
| Tensor parallel | AllReduce (NCCL) | Partial layer outputs | Every layer, every token |
| Pipeline parallel | Send/Recv | Activations between stages | Every token, between stage GPUs |
| Disaggregated prefill/decode | KV-cache transfer (NIXL/UCX) | Full KV-cache (potentially GBs) | Once after prefill completes |

## 11.7 Disaggregated Serving

Modern production serving splits prefill (processing the prompt) and decode (generating tokens) onto separate GPU pools because they have very different compute profiles:

PREFILL POOL (GPU 0, 1):

Process all input tokens in parallel → compute-heavy, high SM utilization

Builds the KV-cache for the entire prompt

↓ KV-CACHE TRANSFER (this is where NIXL shines) ↓

DECODE POOL (GPU 2, 3):

Receives the KV-cache

Generates tokens one at a time → memory-bound, lower SM utilization

Each new token appends K,V to the cache

The KV-cache can also overflow across memory tiers as context grows:

| Tier | Medium | Bandwidth | When Used |
| --- | --- | --- | --- |
| Tier 1 | GPU HBM (VRAM) | ~2 TB/s | Default — fastest, limited capacity |
| Tier 2 | CPU DRAM | ~100 GB/s | KV-cache spills when VRAM full |
| Tier 3 | NVMe SSD | ~7 GB/s | Very long contexts, archival |

NIXL provides a single API to move KV-cache across all three tiers, automatically choosing the optimal backend (NVLink, RDMA, GPUDirect Storage) based on where the data lives.

# 12. GPU Monitoring (nvidia-smi dmon)

Real-time GPU monitoring is essential for understanding whether your training or inference workload is actually utilizing the hardware you're paying for.

## 12.1 nvidia-smi dmon (Live Device Monitoring)

The most useful real-time monitoring command. Run it in a separate terminal while your workload executes:

nvidia-smi dmon -d 1    # Sample every 1 second

nvidia-smi dmon -c 20   # Collect 20 samples then stop

## 12.2 dmon Column Reference

| Column | Metric | What It Tells You |
| --- | --- | --- |
| gpu (Idx) | GPU index | Which GPU (0, 1, 2, ...) |
| pwr (W) | Power draw | Idle: 20-30W. Training: 280-350W. Low = GPU underutilized |
| gtemp (°C) | GPU temperature | Idle: 20-30°C. Load: 60-85°C. Throttles ~90°C |
| sm (%) | SM utilization | % of streaming multiprocessors active. Target: >80% for training |
| mem (%) | Memory controller util | How busy the memory bus is (not VRAM usage) |
| enc/dec (%) | Video encoder/decoder | Only relevant for video workloads, 0% for ML |
| mclk (MHz) | Memory clock | Idle: ~405 MHz. Full speed: ~9501 MHz (GDDR6X) |
| pclk (MHz) | GPU core clock | Idle: ~210 MHz. Full speed: ~1695-2010 MHz |

## 12.3 Reading dmon Patterns

Idle GPUs (no workload running):

# gpu  pwr  gtemp  sm  mem  mclk   pclk

0   32    21    0    0   405    210    ← everything at minimum

1   26    20    0    0   405    210

Training burst (DDP AllReduce benchmark):

0  284    41   99   59  9501   1950   ← both GPUs maxed

1  306    32   99   59  9501   1995

Inference (vLLM single request — bursty, one token at a time):

0  142    31   43   29  9501   1965   ← moderate, asymmetric

1  155    26   24   16  9501   2010

Inference under load (10 concurrent requests):

0  293    38  100   58  9501   1950   ← fully saturated

1  309    30   99   58  9501   1995

## 12.4 What to Watch For

- sm=0% during training: GPU is idle, likely waiting on data loading or CPU bottleneck

- sm=100% but slow throughput: memory bandwidth bound (check mem% column)

- High pwr but low sm%: GPU is doing memory transfers, not compute (bad AllReduce path)

- mclk at idle speed during training: GPU memory not being accessed (data not on GPU)

- Asymmetric load across GPUs: tensor parallel split is unbalanced, or one GPU is doing host work

## 12.5 Other Monitoring Tools

watch -n 1 nvidia-smi           # Refreshes full nvidia-smi every second

nvtop                            # Interactive htop-like GPU monitor

nvidia-smi --query-gpu=utilization.gpu,memory.used,power.draw --format=csv -l 1

# 13. vLLM Inference Under Load

Understanding how GPU utilization behaves under different inference loads helps you right-size your deployment and identify bottlenecks.

## 13.1 Load Testing Setup

Terminal 1 — Start vLLM server:

vllm serve TinyLlama/TinyLlama-1.1B-Chat-v1.0 \

--tensor-parallel-size 2 --dtype float16 --max-model-len 2048 --port 8000

Terminal 2 — Monitor GPUs:

nvidia-smi dmon -d 1

Terminal 3 — Send requests:

# Single request (light load)

curl -s http://localhost:8000/v1/chat/completions \

-H 'Content-Type: application/json' \

-d '{"model":"TinyLlama/TinyLlama-1.1B-Chat-v1.0",

"messages":[{"role":"user","content":"What is NVLink?"}],

"max_tokens":200}'

# Concurrent burst (heavy load)

for i in $(seq 1 10); do

curl -s http://localhost:8000/v1/chat/completions \

-H 'Content-Type: application/json' \

-d '{"model":"TinyLlama/TinyLlama-1.1B-Chat-v1.0",

"messages":[{"role":"user","content":"Write a long essay about distributed computing"}],

"max_tokens":500}' &

done

wait

## 13.2 Measured Utilization Patterns (2x RTX 3090, TinyLlama 1.1B)

| Scenario | SM Util | Power | Mem Ctrl | Pattern |
| --- | --- | --- | --- | --- |
| Server idle | 0% | 110-115W | 0% | Both GPUs at minimum clocks |
| Single request | 24-51% | 140-155W | 16-33% | Bursty, asymmetric across GPUs |
| 10 concurrent requests | 99-100% | 284-309W | 53-59% | Both GPUs fully saturated |
| Between request batches | 0% | 115-135W | 0% | Returns to idle quickly |

## 13.3 Why Inference is Bursty

LLM inference is autoregressive: each token depends on all previous tokens. The GPU spikes during each token generation step, then briefly idles while the result is prepared for the next step. This creates a sawtooth pattern in SM utilization.

With concurrent requests, vLLM batches them together. Instead of generating one token for one request, it generates one token for all 10 requests simultaneously — this is why concurrent load pushes SM utilization to 99-100% while a single request only hits 24-51%.

## 13.4 Tensor Parallelism Observations

- Both GPUs show similar utilization under heavy load (balanced split)

- Under light load, utilization is asymmetric (different layers have different compute costs)

- Memory clocks stay at full speed (9501 MHz) even when idle because model weights remain in VRAM

- Power draws up to 309W under concurrent load (88% of 350W TDP) vs 30W at idle

- NVLink AllReduce happens during every forward pass to combine partial layer outputs

## 13.5 Capacity Planning Takeaways

- Single request uses <50% of your GPU — you have headroom for concurrent serving

- 10 concurrent requests fully saturate 2x 3090 — beyond this, requests queue

- For higher throughput: increase tensor-parallel-size (more GPUs) or use a smaller/quantized model

- Monitor sm% and power to determine when you need to scale: sustained >90% means at capacity

# Appendix: Quick Reference Card

## A.1 Complete Profiling Workflow

# 1. Profile

nsys profile --trace=cuda,nvtx,osrt --pytorch=autograd-shapes-nvtx \

--force-overwrite=true --output=my_profile \

torchrun --nproc_per_node=2 train.py

# 2. Analyze (headless)

nsys stats --report cuda_gpu_kern_sum my_profile.nsys-rep

nsys stats --report cuda_api_sum my_profile.nsys-rep

# 3. Visualize (on laptop)

scp server:my_profile.nsys-rep .

# Open in Nsight Systems GUI (free download from NVIDIA)

## A.2 Hardware Tested

| Component | Specification |
| --- | --- |
| GPU | 2x NVIDIA GeForce RTX 3090 (24GB GDDR6X each) |
| Interconnect | NVLink: 4 links x 14 GB/s = 56 GB/s per direction |
| CPU | AMD EPYC 7742 (128 cores) |
| RAM | 258 GB |
| CUDA | 13.1 |
| Driver | 580.95.05 |
| PyTorch | 2.x with NCCL + Gloo backends |
| Nsight Systems | 2025.1.1 |
| Platform | Vast.ai (Saskatchewan, CA) |